Research Proposal

4-Channel Early Fusion for Real-Time Road Damage Detection

Morphological Attention via Black-Hat Channel Augmentation on YOLOv8

# 1. Problem Statement

Road infrastructure damage — cracks, potholes, and surface deformation — causes significant economic and safety consequences globally. Automated detection using computer vision is a well-studied problem, but a critical gap remains: existing deep learning models either achieve high accuracy using slow segmentation architectures (U-Net, Mask R-CNN) or achieve real-time speeds using lightweight detectors (YOLO) but miss subtle, low-contrast damage like thin longitudinal cracks.

No existing work simultaneously addresses both constraints within a single architecture.

# 2. Core Idea: 4-Channel Early Fusion

We propose a minimal modification to the YOLOv8 input pipeline that augments the standard 3-channel RGB input with a 4th channel derived from classical morphological image processing — specifically the Black-Hat transform.

## What is the Black-Hat Transform?

The Black-Hat operation is defined as:

Black-Hat(I) = Closing(I) − I

It isolates regions that are darker than their local surroundings — precisely the signature of thin cracks on light road surfaces. The result is a soft spatial attention map highlighting where damage is likely to exist.

## The Early Fusion Architecture

Rather than using the Black-Hat mask as a preprocessing filter (which modifies or replaces the original image), we fuse it as an additional input channel alongside the intact RGB data:

Input Tensor: [R, G, B, BlackHat]  →  Modified YOLOv8 Conv1 (4-channel)

The first convolutional layer of YOLOv8 is modified from 3 to 4 input channels. The first 3 channels are initialized from ImageNet pretrained weights; the 4th channel is initialized from the grayscale-equivalent weights, giving it a meaningful starting prior.

# 3. Why This is Novel

Recent literature (2025) has explored hybrid approaches, but none match this specific combination:

| Paper | Approach | Gap |
| --- | --- | --- |
| CrackSense + U-Net (Mar 2025) | Preprocessing → modified RGB → U-Net | Sequential: loses original signal; too slow for real-time |
| MaskerTransformer (Jan 2025) | Mask R-CNN + Vision Transformer | Heavy architecture; not suitable for edge/dashcam deployment |
| This Work | RGB + BlackHat → 4-ch YOLOv8 (Early Fusion) | Preserves full original signal; real-time capable; single-stage |

# 4. The Information-Theoretic Argument

A key theoretical claim of this work is grounded in information theory:

- Prior approaches: Sequential preprocessing pipelines modify the original RGB before feeding it to the network. Any such transformation is lossy unless perfectly invertible.

- Our approach: Our input is a strict superset — we feed the original, unmodified RGB plus the derived mask. The network therefore has access to at least as much information as any preprocessing-only pipeline starting from the same source image.

- Important caveat: More input information does not automatically yield better performance — but it establishes an upper bound. The empirical ablation studies confirm whether the network exploits this additional channel effectively.

# 5. Experimental Plan

## Dataset

- CRDDC2022 (Crowd-sourced Road Damage Detection Challenge 2022) — multi-country dataset with 4 damage classes: longitudinal cracks (D10), transverse cracks (D20), alligator cracking (D40), and potholes (D40)

## Ablation Studies

| Experiment | Purpose |
| --- | --- |
| YOLOv8s baseline (3-channel RGB) | Control — standard model |
| YOLOv8s with 4-channel input (k=9) | Small kernel — sensitive to hairline cracks |
| YOLOv8s with 4-channel input (k=17) | Medium kernel — standard cracks |
| YOLOv8s with 4-channel input (k=25) | Large kernel — wide damage, alligator patterns |
| YOLOv8s with Gaussian blur before BlackHat | Effect of noise suppression on mask quality |

## Evaluation Metrics

- Overall mAP@0.5 and mAP@0.5:0.95

- Per-class AP — expected gains primarily on D10 (longitudinal) and D20 (transverse)

- End-to-end inference FPS including preprocessing latency

- Qualitative visualization of 4th channel activations on hard examples

# 6. Expected Contributions

- Technical: A minimal, architecture-agnostic input modification applicable to any YOLO variant

- Theoretical: Formal information-theoretic justification for early fusion over sequential preprocessing

- Empirical: Empirical evidence that classical morphological priors improve fine-grained crack detection in a fast single-stage detector

- Practical: A kernel-size ablation study characterizing the sensitivity of the Black-Hat channel to different damage scales

# 7. Target Venue

We are targeting a computer vision conference such as WACV (Winter Conference on Applications of Computer Vision) or IEEE IV (Intelligent Vehicles Symposium), where the combination of practical real-world application and a clean novel contribution aligns well with the review criteria.

This document summarizes ongoing research. Code implementation is in progress using the Ultralytics YOLOv8 framework and OpenCV for morphological preprocessing.